# Hazardous Materials Inspection Checklist

This inspection checklist may be used when picking up cars at a customer plant, interchange, or while enroute.

For each car carrying Hazmat, check the following items.  If you notice any problem, immediately contact your supervisor, the Train Dispatcher or the Yard Coordinator.

| # | Inspection Items |
| --- | --- |
| 1 | Inspect the car (from ground level) for: |
|  | - Proper placarding - Placards on each side and end of car/container - Placards visible and legible - Placards correspond to the shipping document and each other - Open or damaged doors, manhole covers and protective housings - Manhole cover swing bolts must be up - Damaged or dragging equipment (running gear, brake system, safety equipment) - Valves and fittings appear to be closed and secure - Plugs or caps on other fittings are securely in place (internal heater coil inlet and outlet pipes may be left open on residue cars to allow them to drain, exterior heater coil pipes need not be capped) - Leakage or venting of the contents - Any suspicious item on or attached to the car/container |
| 2 | Check shipping document for: |
|  | - Car initials and number - Description of the hazmat, in the following order: a) Shipping name b) Technical name (if the product is N. O. S.) c) Class d) Compatibility group (when product is Class 1 – Explosive) e) Subsidiary class (when product has one) f) UN number g) Packing group (I, II or III when product has one; class 2 gasses do not) h) Risk group (A, B, C or D when product is Class 2.3 Toxic Gas or 6.2 – Infectious Substance) - Quantity (weight or volume). - 24-hour emergency response number - Residue information - Additional information that may be present - RQ (when the material is a hazardous substance) - Marine pollutant indication (when product is a marine pollutant) - Toxic Inhalation Hazard (when required) - Packaging and Transport information when product is a Class 7 – Radioactive Material |